A GREEN ROUTE TO TRANSITION METAL-BASED NANOPARTICLES VIA ALUMINIUM CEMENTATION FOR OER ELECTROCATALYSIS

Domenico Lentini1-2, Francesko Malaj2-3, Alessandro Tampucci2, Carlo Santoro3, Enrico Berretti4, Stiven Forti5, Antonio Rossi5, Oreste Tarallo1, Francesco Ruffo1

1 Department of Chemical Sciences, University of Naples Federico II, Italy

2 Ne.m.e.sys. s.r.l., Italy

3 Department of Materials Science, University of Milano-Bicocca, Italy

4 Istituto di Chimica Dei Composti OrganoMetallici, Consiglio Nazionale Delle Ricerche, Italy

5 Center for Nanotechnology Innovation, Istituto Italiano di Tecnologia, Italy
domenico.lentini@unina.it

Anion Exchange Membrane Water Electrolyzers (AEMWEs) offer a promising route for sustainable hydrogen production[1]. To support their widespread deployment, there is a pressing need for efficient, scalable, and environmentally friendly synthesis of cost-effective electrocatalysts[2]. Here, FeCo₃/FeCoOx and Ni₁₋ₓCoₓ/NiCoOx nanoparticles were synthesized via a two-step aqueous synthesis based on aluminium powder cementation, followed by its alkaline removal (Figure 1). This approach integrates principles of green chemistry, including the use of benign solvents, minimal by-products, and low-cost raw materials under mild conditions.

Comprehensive structural and surface characterization was conducted using HRTEM, HAADF-STEM, XPS, XRD, XRF, and SEM. Electrochemical performance was evaluated by LSV, EIS and tests in a 50 cm² pilot-scale AEMWE cell. As OER electrocatalysts, FeCo₃/FeCoOx and Ni₁₋ₓCoₓ/NiCoOx achieved overpotentials of 288 mV and 296 mV at 10 mA cm⁻², and in AEMWE reached 2.10 V and 2.24 V at 1 A cm⁻² (50 °C), respectively. Both outperformed commercial RuO₂ benchmark in all electrochemical experiments. Stability tests over 100 h in AEMWE cell showed minimal degradation, with Ni₁₋ₓCoₓ/NiCoOx maintaining performance and FeCo₃/FeCoOx exhibiting only 396 µV h⁻¹ degradation.

Figure 1: Process diagram

[1]	I.Vincent and D.Bessarabov, Renew. Sustain. Energy Rev., 2018, vol. 81, pp. 1690–1704

[2]	A. Vazhayil, L. Vazhayal, J. Thomas, S. Ashok C, and N. Thomas, Appl. Surf. Sci. Adv., 2021 vol. 6, p. 100184.